BIRZEIT UNIVERSITY

Physics Department

## Physics 112

Experiment No. 6

Capacitor & Inductors

………………………………………………………………………………………

Student’s Name:  Mohammad Sheikh   Student’s No.:  1221541

Partner’s Name:   Mousa Suhaib           Partners’ No.:  1210143

Instructor: Shayma’ Salama                 Section No.:  6

Date:	  18-12-2023

- Abstract :

Introduction : In this experiment, it is expected to discover the time constant of three electrical circuits: RC-circuit, LR-circuit, and LC-circuit. To briefly explain, RC-circuit is an electrical circuit containing resistance, which is connected to a capacitor of capacitance. Secondly, LR-circuits would be where the capacitance of inductance, L, is joined in series with a capacitance of resistance, R. Lastly, the LC-circuit is formed when an inductor, L, and capacitor, C, are joined together. Therefore, to find the time constant of the three electrical circuits would be to use a DCO to measure the voltage of RC and RL circuits, as well as find the frequency of the LC current.

The aim of the experiment : is to find out the time constant in RC, RL ,

and LC-circuits.

The method used : is by using the DCO to measure the voltage in the

RC and LR circuits and to measure the frequency in the LC circuit.

The main result :

- RC circuits :

τc  = 104 µsec

τd  = 104 µsec

τexp  = 104 µsec

τtheo  = 100 µsec

- LR circuit :

τc  = 9 µsec

τd  = 9 µsec

τexp  = 9 µsec

τtheo  = 10 µsec

- LC circuit :

fexp  = 4.63  KHz

ftheo = 5 KHz

ωexp  = 29.1 rad/s

ωtheo = 31.4 rad/s

- Theory :

RC circuits :

Charging a capacitor :

The voltage across the capacitor’s plates is defined by  VC = QC  , and

While Q(t) = Cϵ ( 1-e - tRC )  (when we are talking about the positive half period of the square wave) , then Vc = ϵ ( 1-e - tRC ) .

.The value of RC is usually called the time constant (τ) of the RC

circuit like the one shown in fig.1. τ is a measure of how fast the voltage across the capacitor rises. When   t= τ   ,  VC = 0.63 ϵ

The current passing through the circuit is given by : I(t) = dQdt = ϵR e - tRC

, while the voltage across the resistor is : VR = I(t) R = ϵe - tRC   .

Discharging a capacitor:

Now, during the negative period of the square wave, the

capacitor, the capacitor discharges according to the following formula :

Q(t) = C ϵe - tRC   ,   And so the voltage across the capacitor’s plates is:

VC = 0.37 ϵ   .

In this case the voltage decays to 0.37 of its maximum value within

a time τ, which equals RC (the time constant).

The current passing through the circuit is:

I(t) =  dQdt = ϵR e - tRC

and so, the voltage is given by:

VR = I(t) R  = - ϵe - tRC

LR circuits:

The current passing through the LR circuit shown in

fig.3 rises with time according to the following equation:

I = ϵR ( 1-e - RtL )

The voltage across the resistor is : I = IR =  ϵ( 1-e - RtL )

and across the inductor is : VL  = L dIdt = ϵe - RtL

In this case the time constant equals LR . When  t = τ  ,   and    VL = 0.37 ϵ   ,

and   VR =  0.63 ϵ .

LC circuits :

The following equation describes the voltage

across the capacitor’s plates of the one in the circuit

shown in fig.5,

VC = VCo cos ( ω t + φ )

Where VCo  is the amplitude (constant ) and  ω = 1LC  .

Fig.6 shows the voltage across the capacitor as a

function of time .

- Data :

R = 1 kΩ                                   C = 0.1 µf                              L = 10 mH

1 ) RC-circuit :

| = 104 µsec | = 104 µsec |  |
| --- | --- | --- |
| = (104+104)/2 = 104 µsec |  | = RC = (1000Ω)×(0.1µf) = 100 µsec |

2) LR-circuit :

| = 9 µsec | = 9 µsec |  |
| --- | --- | --- |
| = (9+9)/2 = 9 µsec |  | = (L/R) = (10 * 10-3) / 103 = 10 µsec |

3 )  LC-circuit

|  | 4.36 KHz | =2πf 29.1 rad/s |
| --- | --- | --- |
|  | = 12π LC = 12π10*10-3 * .01 * 10-6 = 5.0 KHz | =2πf 31.4 rad/s |

- Discussion & Conclusion :

This experiment discussed the three different circuits: RC-circuit, LR-circuit, and LC-circuit that were displayed on the DSO, where their times experimentally and theoretically were measured. With the addition of frequency being measured experimentally and theoretically in the LC-circuit. However, as shown in the data section, many of the results received from the DSO (experimental) were not similar or accurate to those calculated (theoretical). The reasoning for such would be because the digital multimeter that was used was not in a state that could display precise results. In addition to this, after using the noise-cancelling wires: it was found that there was still noise displayed on the DSO. Therefore, the reasoning for such different experimental results: would be that the tools were inaccurate and unable to measure correctly. As a result, the DSO displayed graphs that were erroneous.